
Powder Drain

for 


STAR Reloader


         by Al Ganser: dba Couragecara  (1 574 825 3181)
Step x Step instruction  to empty powder hopper and  powder drain.
1. turn 3/4" alum knob to the left

2. back off 1/16" (coded red ) set screw.
3. pivot powder hopper  with 1/4 alum plate to left 

4. Lift powder hopper & drain off of brass coupling

5. Pour un-used powder back into container

To re-install powder hopper and drain:

1.place hopper & drain onto the brass coupling

2. tighten 1/16" (coded red ) set screw

3. Rotate entire hopper/drain assembly to the far right and tighten 3/4 " alum knob, this removes any 'wiggle', making all parts  rigid. 

tools needed: 

1. 5/32 hex wrench for set screw on bottom of pwd drain. This set screw holds the two disc together. Recommend it be left as is.
2.1/16"  hex wrench for set screw (coded red) on top of 1/4" x 1 3/4" alum disc.

Note: set screw coded red is the only set screw used   for normal on and off operation. The other set screws are not needed unless dis-assembly is needed.

ARH 2-28-2014

   .

